SMK Bridge Track — ICE Fundamentals Pre-Requisite

Instructor Manual (6 Weeks / 30 Sessions) — Companion to the 12-Week EV Diagnosis & Repair Program

# Why This Track Exists

This is not a shortened version of the main 12-week EV program. It is a separate, purpose-built pre-requisite for cohorts who don't yet have ICE-diagnostic-technician-level skills — most notably SMK (Indonesian vocational high school) students, who typically have some ICE fundamentals from coursework but are not yet working diagnostic technicians.

This version was expanded from 4 to 6 weeks (20 to 30 sessions) based on field expert feedback, which recommended a more thorough electrical fundamentals foundation (series/parallel circuits, electromagnetism, diode rectification, transformers, thermistors) plus basic automotive mechanical fundamentals (gear ratio, torque vs. power, cooling/heat management) — content that a 50-question placement test typically expects.

Do not use this track for cohorts that already meet the ICE-technician prerequisite (e.g. Toyota-style novice technicians already employed post-training) — for that audience, start directly at Week 1 of the main EV program; this bridge track would be redundant.

Rule of thumb: if a cohort has already worked as a paid or apprenticed automotive technician, skip this bridge track. If they are current students without professional technician experience, run this track first — and use the 50-question placement test to confirm.

# Structure at a Glance

| Week | Focus | Why It Matters Later |
| --- | --- | --- |
| B1 | Vehicle systems, shop safety, documentation, Ohm's Law | Every later week assumes this vocabulary and math without re-explaining it |
| B2 | Series/parallel circuits, electromagnetism, diodes, transformers, thermistors | Directly underlies EV power electronics (inverters, chargers) and sensor systems |
| B3 | 4-step diagnostic pattern, multimeter use, DTC basics | This IS the diagnostic thinking pattern used throughout the entire 12-week EV program |
| B4 | Gear ratio, torque vs. power, ICE cooling/heat management | Reappears directly in EV drivetrain and battery thermal management content |
| B5 | 12V battery/charging, wiring diagrams, sensor-actuator-ECU | The sensor-ECU-actuator loop is exactly the EV program's BMS concept, just at low voltage first |
| B6 | Documentation, communication, safety culture, readiness gate | Confirms genuine readiness before Week 1 of the main program, not just completion |

# The Week B6 Day 5 Readiness Gate — Read This Carefully

This is the most important instructional decision point in the bridge track. Day 5 is not a formality — it is an honest checkpoint.

If a participant is not genuinely confident in one or more of the 6 bridge weeks, the correct move is to have them repeat that specific week, not to push them into Week 1 of the main EV program. This is the exact same "a weak item takes priority over pace" principle used throughout the main program's OJT phase (Weeks 9-12) — apply it here too.

A participant who begins Week 1 of the main EV program without a solid foundation in all 6 bridge weeks will experience compounding confusion, because Week 1 onward assumes this material without re-explaining it. Letting someone through the gate as a courtesy does them a disservice.

# The 50-Question Placement Test

Before assigning a cohort to this bridge track at all, use placement-test.html — a 50-question quiz covering all 6 weeks (roughly 8-9 questions per week). A score of 80%+ means the participant can skip directly to Week 1 of the main program. 50-79% suggests completing the bridge track with focus on the weak section(s) the test identifies. Below 50% suggests starting from Week B1.

# Session-by-Session Scripts (30 Sessions)

## Week B1 — Vehicle Systems & Shop Fundamentals

Goal: build the baseline every later week assumes — how a vehicle's systems fit together, general shop safety, reading documentation, and basic electrical math.

### Week B1 · Day 1 — How a Vehicle Works — the Big Systems Picture

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"Before diagnosing anything, you need a mental map of what's actually in a car and how the pieces talk to each other."

Theory Script

- 1. A vehicle is made of major systems that interact: engine/powertrain, electrical, chassis/suspension, braking, and body/interior.

- 2. Diagnostic thinking always starts by asking "which system is this symptom coming from?" before going any deeper.

- 3. Most modern vehicles also have a central computer (ECU) that many systems report to — this becomes very important later.

- 4. A technician's job is rarely to fix one bolt — it's to correctly identify which system owns the problem first.

- ECU = Electronic Control Unit — the vehicle's onboard computer that monitors and controls many systems.

Example/analogy: "Analogy: think of a car like a hospital. The ECU is the reception desk that routes a patient (a symptom) to the right department (system) — engine, electrical, brakes, etc."

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A customer says the car "just feels weak" when accelerating, no warning lights.

A. Loss of power under acceleration points to the engine/powertrain system first.

### Week B1 · Day 2 — Hand Tools & General Shop Safety

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Select

Opening

"Before touching any vehicle, a shop has non-negotiable safety habits — this is the foundation Week 1 of the EV program builds on."

Theory Script

- 1. Always wear safety glasses and closed-toe shoes in the shop — no exceptions, regardless of the task.

- 2. Use the correct tool for the job — using the wrong size wrench is a leading cause of stripped bolts and hand injuries.

- 3. Keep your workspace organized — tools left on the floor or engine bay are a tripping/dropping hazard.

- 4. Never work under a vehicle that is not properly supported on rated jack stands — a jack alone is not a support device.

- Jack stand = A mechanical support device rated to hold a vehicle's weight safely — different from a jack, which only lifts.

Simulator Facilitation

How it works: Multiple items are shown; the participant selects the correct one(s) and submits.

Tip: Tell them not to submit until confident — this type sees a lot of rushed, wrong submissions.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A classmate is about to slide under a car that's held up by a jack only, no stands.

A. A jack alone can fail — jack stands are mandatory before anyone goes underneath.

### Week B1 · Day 3 — Reading a Service Manual & Technical Documentation

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Match

Opening

"Every later week in this program assumes you can find and read the right page in a manual — this is that skill, explicitly."

Theory Script

- 1. A service manual is organized by system (engine, electrical, brakes, etc.) — knowing the system name from Day 1 is what lets you find the right chapter fast.

- 2. Manuals use standardized symbols and abbreviations — learning to recognize a handful of common ones saves huge amounts of time.

- 3. Specification tables (torque values, resistance ranges, fluid capacities) are the most-referenced part of any manual — always confirm against them rather than guessing.

- 4. When in doubt about a spec, the manual is always right over memory or assumption.

- Specification (spec) = An exact value (e.g. torque, resistance, capacity) that a manufacturer requires for a correct repair.

Example/analogy: "Analogy: a service manual is like a recipe book — skipping the exact measurements (specs) is how recipes (repairs) go wrong."

Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

Tip: No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. You're not sure of the exact torque value for a bolt, but you remember roughly what a similar bolt needed.

A. Always confirm exact specs in the manual rather than relying on memory.

### Week B1 · Day 4 — Basic Electrical Fundamentals — Voltage, Current, Resistance

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Meter Read

Opening

"This is the single most important day in the bridge track — every EV week later assumes you're comfortable with this math."

Theory Script

- 1. Voltage (V) is electrical pressure — what pushes current through a circuit. Measured in volts.

- 2. Current (A) is the flow of electricity — measured in amps. Current only flows when there's a complete circuit.

- 3. Resistance (Ω) opposes current flow — measured in ohms. More resistance means less current for the same voltage.

- 4. Ohm's Law ties them together: Voltage = Current × Resistance (V = I × R). This one formula underlies almost every electrical diagnostic calculation you'll do later.

- Ohm's Law = V = I × R. Voltage equals current multiplied by resistance — the foundational formula of electrical diagnosis.

Example/analogy: "Analogy: voltage is like water pressure, current is like water flow rate, and resistance is like how narrow the pipe is. A narrower pipe (more resistance) means less flow (current) for the same pressure (voltage)."

Simulator Facilitation

How it works: Several readings are shown; the participant judges each as safe/normal or not.

Tip: Getting the right answer matters less than making them explain WHY — have them recompute or reason it through themselves.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A circuit shows 24V and you measure 2A of current flowing.

A. R = V/I = 24V / 2A = 12Ω.

### Week B1 · Day 5 — Week B1 Wrap-Up — Systems & Safety Review

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"Before moving to diagnostic reasoning, let's confirm this week's foundation is solid."

Theory Script

- 1. This week covered: vehicle systems overview, shop safety, reading documentation, and Ohm's Law.

- 2. Every one of these four topics gets used, without re-explanation, starting next week — this is the last checkpoint before the pace picks up.

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A new student asks you why Ohm's Law matters for a car that isn't even electric.

A. Every vehicle has electrical systems (starter, lights, sensors) that follow Ohm's Law, not just EVs.

## Week B2 — Circuit & Electromagnetic Fundamentals

Goal: extend Week B1's Ohm's Law into real circuit configurations, and build the electromagnetism, rectification, transformer, and thermistor knowledge that EV power electronics and sensor systems are built on.

### Week B2 · Day 1 — Series & Parallel Circuits

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"Real circuits are almost never just one resistor — components are wired in series, parallel, or a mix of both, and each configuration behaves differently."

Theory Script

- 1. In a series circuit, components share one single path — the same current flows through all of them, but voltage divides across each one.

- 2. In a parallel circuit, components each have their own path back to the source — voltage is the same across each branch, but current divides between them.

- 3. Series total resistance: simply add all resistances together (R_total = R1 + R2 + R3...).

- 4. Parallel total resistance is always LESS than the smallest individual resistor — more paths means less overall opposition to current flow.

- Series circuit = Components connected in a single path — same current throughout, voltage divides.

- Parallel circuit = Components each with their own path — same voltage across each branch, current divides.

Example/analogy: "Analogy: series is like one single-lane road — everyone shares the same lane. Parallel is like several lanes side by side — traffic (current) splits between them, but the destination (voltage) is the same for each lane."

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. Two identical resistors are wired in parallel across a 12V source.

A. Adding a parallel path always lowers total resistance — for two identical resistors, it's exactly half.

### Week B2 · Day 2 — Fundamentals of Electromagnetism

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Match

Opening

"Electromagnetism is the physical principle behind motors, generators, alternators, and relays — nearly every moving electrical part in a vehicle relies on it."

Theory Script

- 1. Whenever current flows through a wire, it creates a magnetic field around that wire — this is the core link between electricity and magnetism.

- 2. Coiling a wire concentrates that magnetic field, and adding an iron core concentrates it further — this is how an electromagnet is built.

- 3. The reverse also works: moving a magnet near a coil of wire induces (creates) a current in that wire — this is how generators and alternators produce electricity.

- 4. A relay uses a small electromagnet to pull a switch closed, letting a small control current switch a much larger power circuit safely.

- Electromagnet = A magnet created by current flowing through a coiled wire, often around an iron core.

- Induction = Generating a current in a wire by moving a magnetic field near it (or moving the wire through a field).

Example/analogy: "Analogy: a relay is like a small person (the control current) who isn't strong enough to lift a heavy gate (the power circuit) — but is strong enough to flip a lever (the electromagnet) that lifts it for them."

Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

Tip: No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A relay clicks when energized but the large device it's supposed to power doesn't turn on.

A. The click confirms the control side (electromagnet) works — the fault is likely on the power switching side of the relay.

### Week B2 · Day 3 — Diode Rectifiers

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Select

Opening

"A diode is one of the simplest but most important electronic components — and rectification (AC to DC conversion) is exactly what an alternator's internal diodes do every time the engine runs."

Theory Script

- 1. A diode allows current to flow in only one direction — forward — and blocks it in the reverse direction, acting like a one-way valve for electricity.

- 2. Rectification is the process of converting AC (alternating current, which reverses direction repeatedly) into DC (direct current, which flows one way) using diodes.

- 3. A vehicle's alternator actually generates AC internally, but a set of diodes (the rectifier) converts it to the DC the battery and vehicle electronics need.

- 4. A failed diode in an alternator's rectifier is a real, common fault — it can cause a whining noise, reduced charging output, or a battery that won't fully charge.

- Diode = A component that allows current to flow in only one direction.

- Rectifier = A set of diodes that converts AC to DC.

Example/analogy: "Analogy: a diode is like a one-way turnstile — people (current) can push through in one direction, but the turnstile physically blocks anyone trying to go backward."

Simulator Facilitation

How it works: Multiple items are shown; the participant selects the correct one(s) and submits.

Tip: Tell them not to submit until confident — this type sees a lot of rushed, wrong submissions.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A vehicle has a whining noise from the alternator area and the battery isn't fully charging.

A. A failed rectifier diode is a real, common cause of alternator charging problems and unusual noise.

### Week B2 · Day 4 — Step-Up / Step-Down Transformers

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"Transformers change voltage levels using nothing but electromagnetism — no moving parts — and EVs rely on this same principle at a much larger scale for charging and onboard power conversion."

Theory Script

- 1. A transformer has two coils (windings) that aren't electrically connected — they transfer energy purely through a shared magnetic field, using the induction principle from Day 2.

- 2. A step-up transformer has more turns on the output coil than the input — it increases voltage (and decreases current proportionally).

- 3. A step-down transformer has fewer turns on the output coil than the input — it decreases voltage (and increases current proportionally).

- 4. Transformers only work with AC, not DC — a constant DC current doesn't create the changing magnetic field induction requires.

- Step-up transformer = Increases voltage from input to output.

- Step-down transformer = Decreases voltage from input to output.

Example/analogy: "Analogy: think of two gears of different sizes connected by a shared chain (the magnetic field) — a small gear driving a big one slows rotation down but multiplies force; a big gear driving a small one speeds rotation up but reduces force. Voltage and current trade off the same way."

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. Someone tries to connect a transformer directly to a constant DC power source instead of AC.

A. Transformers rely on a constantly changing magnetic field, which only AC provides — constant DC doesn't induce output voltage.

### Week B2 · Day 5 — Thermistors & Week B2 Wrap-Up

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"A thermistor is one of the most common sensors in any vehicle — and this session both introduces it and reviews everything from this week."

Theory Script

- 1. A thermistor is a resistor whose resistance changes predictably with temperature — this makes it useful as a temperature sensor.

- 2. Most automotive thermistors are NTC (Negative Temperature Coefficient) — resistance goes DOWN as temperature goes UP.

- 3. The ECU reads a thermistor's resistance (via voltage) and converts it to a temperature reading, using a known resistance-vs-temperature curve.

- 4. A thermistor reading a wrong or stuck value is a common real fault — recall from Week B3's sensor/actuator content that a wrong sensor input can cause the ECU to make a wrong decision.

- Thermistor = A resistor whose resistance changes predictably with temperature, used as a temperature sensor.

- NTC = Negative Temperature Coefficient — resistance decreases as temperature increases; the common type in automotive use.

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. An NTC thermistor's resistance reading is unusually HIGH for an engine that's clearly warmed up.

A. For an NTC thermistor, high resistance normally means low temperature — a mismatch with a known-warm engine is a red flag worth investigating.

## Week B3 — Basic Diagnostic Reasoning

Goal: build the symptom→cause→verify thinking pattern that every later diagnostic week in the main program assumes you already have.

### Week B3 · Day 1 — What Is Diagnosis? Symptom → Cause → Verify

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sequence

Opening

"Diagnosis isn't guessing — it's a repeatable thinking pattern. Today you learn the pattern itself, before any specific system."

Theory Script

- 1. Step 1 — Symptom: what does the customer or data actually report? Get this precise before anything else.

- 2. Step 2 — Possible causes: list plausible causes for that exact symptom, from most to least likely.

- 3. Step 3 — Test: check the most likely, lowest-cost cause first — don't jump to the expensive part.

- 4. Step 4 — Verify: after a fix, confirm the original symptom is actually gone, not just assumed fixed.

- Low-cost check = A cheap, fast test performed before expensive or time-consuming ones — check simple things first.

Example/analogy: "Analogy: a doctor doesn't order surgery before checking your temperature and asking what hurts. Diagnosis always starts cheap and simple, then narrows down."

Simulator Facilitation

How it works: Show the items in random order; the participant clicks them one at a time in the order they believe is correct.

Tip: Ask "why that order?" before revealing the answer on a wrong attempt — repeat until 3 flawless runs in a row.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A customer says "the car makes a weird noise sometimes."

A. A vague symptom must be made precise before you can list plausible causes.

### Week B3 · Day 2 — Using a Multimeter Safely (12V Practice)

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sequence

Opening

"The multimeter is the single most-used diagnostic tool — and this 12V practice is what makes the EV program's later HV meter work feel familiar instead of brand new."

Theory Script

- 1. A multimeter can measure voltage, current, and resistance — always set it to the correct mode before connecting it.

- 2. For voltage measurement, connect the meter in parallel (across the two points) — never in series.

- 3. Always start on the highest range setting if unsure of the expected value, then narrow down — this protects the meter from damage.

- 4. Confirm the meter itself is working (test on a known source) before trusting a 'no reading' result — it might be a broken meter, not a broken circuit.

- Parallel connection = Connecting a meter across two points of a circuit to measure voltage, without breaking the circuit.

Simulator Facilitation

How it works: Show the items in random order; the participant clicks them one at a time in the order they believe is correct.

Tip: Ask "why that order?" before revealing the answer on a wrong attempt — repeat until 3 flawless runs in a row.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. You connect a multimeter set to voltage mode and get no reading (0V) on a circuit you expected to have 12V.

A. Always confirm the meter is working before trusting an unexpected reading.

### Week B3 · Day 3 — Reading OBD-II / DTC Codes (ICE-Level Introduction)

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Match

Opening

"Diagnostic trouble codes exist on every modern vehicle, not just EVs — this is the gentle on-ramp before the EV program's own DTC system."

Theory Script

- 1. A DTC (Diagnostic Trouble Code) is a code the ECU stores when it detects something outside normal range.

- 2. Codes are typically structured by letter + numbers — the letter tells you which system (e.g. P for Powertrain).

- 3. A code tells you where the ECU noticed a problem — it does NOT automatically tell you the root cause. Further diagnosis is still required.

- 4. Clearing a code without fixing the underlying cause means it will very likely come back.

- DTC = Diagnostic Trouble Code — a stored code indicating the ECU detected a reading outside normal range in a specific system.

Example/analogy: "Analogy: a DTC is like a smoke alarm going off — it tells you where the smoke is, not necessarily what's burning. You still have to go look."

Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

Tip: No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A technician reads a DTC, then immediately clears it without further checks and tells the customer it's fixed.

A. A code is a starting point for diagnosis, not a fix — the underlying cause must still be found and corrected.

### Week B3 · Day 4 — Basic Mechanical Fault-Finding Practice

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Branch

Opening

"Combining the 4-step diagnostic pattern with a real (simple) mechanical scenario."

Theory Script

- 1. Mechanical symptoms often show a pattern: constant vs. intermittent, and load-dependent vs. not — noting this narrows causes fast.

- 2. Visual inspection (leaks, loose parts, wear marks) is almost always a valid low-cost first check before any tool comes out.

- 3. Comparing to a known-good reference (another vehicle, a spec sheet) helps you recognize what 'abnormal' actually looks like.

Simulator Facilitation

How it works: A situation is presented; choosing an option leads to the next situation and feedback.

Tip: Get them fully immersed as "the technician." Ask what made them choose that option before reading the feedback.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A noise happens constantly, regardless of speed, load, or road surface.

A. A constant, condition-independent noise rules out bump- or load-triggered causes, redirecting your search.

### Week B3 · Day 5 — Week B2 Wrap-Up — Diagnostic Reasoning Review

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"This week's thinking pattern gets reused, unexplained, in every diagnostic week from here forward — let's confirm it's solid."

Theory Script

- 1. This week covered: the 4-step diagnostic pattern, safe multimeter use, DTC code basics, and applying it all to a mechanical scenario.

- 2. Notice that none of this was EV-specific — that's intentional. This is the general skill the EV-specific weeks build on top of.

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A classmate says DTC codes make diagnosis unnecessary since "the code tells you what's wrong."

A. This is a common misconception worth correcting clearly.

## Week B4 — Automotive Mechanical Fundamentals

Goal: build the core mechanical concepts (gear ratio, torque vs. power, cooling/heat management) that apply to every vehicle drivetrain and thermal system, ICE or EV.

### Week B4 · Day 1 — Gear Ratio Fundamentals

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"Gear ratios are why a vehicle has multiple gears at all — and the same math applies whether it's a manual transmission or an EV's single-speed reduction gear."

Theory Script

- 1. Gear ratio compares the number of teeth (or rotations) on a driving gear versus a driven gear — expressed as driven:driving or as a single number.

- 2. A HIGH gear ratio (e.g. low gear, like 1st gear) means the driving gear turns many times for one turn of the driven gear — this multiplies torque but reduces speed.

- 3. A LOW gear ratio (e.g. high gear, like 5th gear or overdrive) means less torque multiplication but higher output speed for the same engine RPM.

- 4. Even EVs use a gear ratio — usually a single fixed reduction gear between the motor and the wheels, since electric motors don't need multiple gears to stay in an efficient RPM range.

- Gear ratio = The ratio of rotations between a driving gear and a driven gear, determining the torque/speed tradeoff.

Example/analogy: "Analogy: pedaling a bicycle in a low gear (easy pedaling, low speed, lots of climbing torque) versus a high gear (hard pedaling, high speed, less torque) — same legs, same effort capacity, completely different output depending on gear choice."

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A vehicle needs maximum torque to climb a steep hill at low speed.

A. A high gear ratio multiplies torque at the cost of speed — exactly what a steep climb needs.

### Week B4 · Day 2 — Torque vs. Power

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Match

Opening

"Torque and power are often confused, but they measure genuinely different things — and this distinction matters even more for EVs, which deliver torque very differently from ICE engines."

Theory Script

- 1. Torque is a twisting/rotational force — it's what accelerates a vehicle from a stop and what a customer feels as "push" when flooring the pedal.

- 2. Power is the RATE of doing work over time — it's related to torque and RPM together (Power = Torque × RPM, with a conversion constant).

- 3. An ICE engine's torque typically builds gradually and peaks at a specific RPM range — it needs to "rev up" to reach peak output.

- 4. An EV motor can deliver maximum torque instantly from 0 RPM — this is why EVs often feel much quicker off the line than their power figures alone would suggest.

- Torque = Rotational/twisting force, measured independent of speed.

- Power = The rate of doing work — depends on both torque and rotational speed (RPM).

Example/analogy: "Analogy: torque is how hard you can push a stuck door; power is how fast you can keep pushing doors like that, one after another. A weightlifter has huge torque (one big push); a sprinter has high power (sustained fast output) — different kinds of "strong.""

Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

Tip: No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A customer says an EV "feels faster" off the line than an ICE car with a higher horsepower rating.

A. Instant torque delivery is a defining EV characteristic, independent of the overall power figure.

### Week B4 · Day 3 — ICE Cooling System & Heat Management

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"Every engine generates enormous heat, and managing it is a full system — the same fundamental cooling logic reappears later for EV battery thermal management."

Theory Script

- 1. An ICE cooling system circulates coolant through the engine block to absorb heat, then through a radiator to release that heat to the outside air.

- 2. The thermostat controls coolant flow based on temperature — staying closed when cold (to let the engine warm up faster) and opening once at operating temperature.

- 3. The water pump (usually belt- or chain-driven) keeps coolant circulating continuously whenever the engine runs.

- 4. Overheating can come from several causes: low coolant, a stuck thermostat, a failed water pump, a clogged radiator, or a failed cooling fan — diagnosis means checking each low-cost item before assuming the worst.

- Thermostat = A valve that controls coolant flow based on temperature, closed when cold and open at operating temperature.

- Radiator = A heat exchanger that releases coolant's absorbed heat to outside air.

Example/analogy: "Analogy: the cooling system is like a delivery route — the water pump is the driver, coolant is the truck, the engine is the pickup point loading heat, and the radiator is the drop-off point unloading it into the air."

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. An engine overheats, and you find the coolant level is actually normal.

A. With coolant level ruled out, checking the thermostat and water pump is the logical next low-cost step.

### Week B4 · Day 4 — Applying Mechanical Concepts Together

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Branch

Opening

"A practice scenario combining gear ratio, torque/power, and cooling system reasoning from this week."

Theory Script

- 1. Real symptoms often touch more than one mechanical concept at once — a vehicle struggling on a hill could involve gear selection, torque delivery, or overheating from sustained load, or some combination.

- 2. The same low-cost-first, symptom-to-cause discipline from Week B3 applies just as much to mechanical reasoning as to electrical.

Simulator Facilitation

How it works: A situation is presented; choosing an option leads to the next situation and feedback.

Tip: Get them fully immersed as "the technician." Ask what made them choose that option before reading the feedback.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. Why does towing a heavy load up a long hill increase overheating risk even if the cooling system is working normally?

A. High sustained load generates more heat, while lower speed reduces radiator airflow — a genuinely higher-risk combination even with a healthy cooling system.

### Week B4 · Day 5 — Week B4 Wrap-Up — Mechanical Fundamentals Review

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"Confirming this week's mechanical foundation before moving into Week B5's electrical systems content."

Theory Script

- 1. This week covered: gear ratio, torque vs. power, ICE cooling/heat management, and applying all three together in a combined scenario.

- 2. These mechanical fundamentals apply almost unchanged when working with EV drivetrains and battery thermal management later in the 12-week main program.

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. Looking back at this week, which concept do you expect will reappear most directly when this program later covers EV battery thermal management?

A. EV battery thermal management uses the same fundamental heat-generation and heat-transfer logic as ICE engine cooling, just applied to a battery pack instead of an engine block.

## Week B5 — Electrical & Electronic Systems

Goal: build comfort with 12V battery/charging systems, wiring diagrams, and the sensor-actuator-ECU relationship — the direct conceptual bridge into the EV program's BMS-heavy content.

### Week B5 · Day 1 — 12V Battery & Charging System Basics

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Meter Read

Opening

"Every ICE vehicle already has a small-scale version of the battery/charging concepts EV Week 2 will build on."

Theory Script

- 1. The 12V battery stores energy and provides high current briefly for starting the engine.

- 2. The alternator recharges the battery and powers electrical systems once the engine is running.

- 3. A battery's state can be judged by voltage: roughly 12.6V+ = healthy resting charge, below ~12.0V = weak/discharged.

- 4. A charging system fault (bad alternator) often shows up as a battery that drains repeatedly, not just once.

- Alternator = A generator driven by the engine that recharges the battery and powers electrical loads while the engine runs.

Simulator Facilitation

How it works: Several readings are shown; the participant judges each as safe/normal or not.

Tip: Getting the right answer matters less than making them explain WHY — have them recompute or reason it through themselves.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A customer's battery keeps going dead every few days, even after being recharged each time.

A. Repeated draining despite recharging points to the charging system, not just the battery itself.

### Week B5 · Day 2 — Reading Wiring Diagrams

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sequence

Opening

"Wiring diagrams look intimidating at first — today breaks them into the handful of symbols that cover most of what you'll see."

Theory Script

- 1. A wiring diagram shows components as symbols connected by lines representing wires, often color-coded.

- 2. Following a diagram means tracing a path from power source, through a component, to ground — a circuit needs all three.

- 3. Wire colors and gauge (thickness) are usually labeled and matter — swapping a thin wire for a thick one's job can cause overheating.

- 4. Ground symbols matter as much as power symbols — a broken ground is a very common, easy-to-miss fault.

- Ground = The return path of a circuit back to the vehicle's chassis/battery negative — required to complete any circuit.

Example/analogy: "Analogy: a circuit is like a water loop — power is the pump pushing water out, the component is a wheel the water turns, and ground is the pipe returning water back to the pump. Break any part and the loop stops."

Simulator Facilitation

How it works: Show the items in random order; the participant clicks them one at a time in the order they believe is correct.

Tip: Ask "why that order?" before revealing the answer on a wrong attempt — repeat until 3 flawless runs in a row.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A component isn't working, and you've confirmed power is reaching it correctly.

A. A confirmed-good power supply doesn't rule out a broken ground — both are required for the circuit.

### Week B5 · Day 3 — Sensors & Actuators — How They Talk to the ECU

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"This is the conceptual bridge day — the EV program's BMS and sensor-heavy content assumes you already understand this relationship."

Theory Script

- 1. A sensor measures something (temperature, speed, position) and reports it to the ECU as an electrical signal.

- 2. An actuator receives a command from the ECU and does something physical (opens a valve, turns a motor, lights an indicator).

- 3. The ECU's job is to read sensors, make a decision based on programmed logic, and command actuators in response.

- 4. If a sensor sends a wrong or missing signal, the ECU can make a wrong decision — this is why sensor faults often cause seemingly unrelated symptoms.

- Sensor = A component that measures a physical condition and converts it into an electrical signal for the ECU.

- Actuator = A component that receives an ECU command and performs a physical action.

Example/analogy: "Analogy: sensors are the ECU's eyes and ears, actuators are its hands. The ECU itself is the brain deciding what to do with what it senses."

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A cooling fan doesn't turn on even though the engine is clearly overheating.

A. Both a sensor fault (wrong input) and an actuator fault (bad output) can cause the same visible symptom — this is why diagnosis matters.

### Week B5 · Day 4 — Combined Electrical + Mechanical Troubleshooting Practice

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"Real-world faults don't announce whether they're electrical or mechanical — today combines everything from Weeks B1-B3."

Theory Script

- 1. Always separate a symptom into possible electrical vs. mechanical causes before testing — this keeps your search organized.

- 2. A quick way to start separating: does the fault change with vibration/movement (often mechanical/connection) or is it constant (often a steady electrical fault)?

- 3. Use the low-cost checks first, then the multimeter, then deeper inspection — the same order regardless of system.

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A gauge that normally moves is now stuck at exactly zero, all the time, regardless of conditions.

A. A hard stop at exactly zero often suggests a broken connection or total signal loss, not a gradual sensor drift.

### Week B5 · Day 5 — Week B3 Wrap-Up — Electrical Systems Review

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Match

Opening

"This week's sensor-actuator-ECU concept in particular will resurface constantly in the EV program's BMS content."

Theory Script

- 1. This week covered: 12V battery/charging basics, wiring diagram reading, the sensor-actuator-ECU relationship, and combined troubleshooting.

- 2. The sensor → ECU → actuator loop from Day 3 is the single most important concept to carry forward — the EV program's BMS is exactly this same loop, just at high voltage.

Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

Tip: No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. Looking back at this week, which single concept do you think will matter most for understanding an EV's BMS later?

A. The BMS is fundamentally the same sensor-decision-actuator loop you learned this week, just applied to high-voltage battery cells.

## Week B6 — Professional Readiness & Gate Check

Goal: build professional habits (documentation, communication, safety culture) and confirm — with an honest gate check — readiness to begin Week 1 of the main 12-week EV program.

### Week B6 · Day 1 — Documentation & Work Orders

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sequence

Opening

"Diagnosis that isn't written down might as well not have happened — professional habits start with recordkeeping."

Theory Script

- 1. A work order should record: the reported symptom, tests performed, findings, and the fix applied — in that order.

- 2. Write findings precisely, not vaguely — "12.1V, below normal" is useful; "battery seemed weak" is not.

- 3. Good documentation protects both the technician and the customer if a problem returns or is disputed later.

- Work order = The formal record of a repair job: reported symptom, diagnostic steps, findings, and repair performed.

Simulator Facilitation

How it works: Show the items in random order; the participant clicks them one at a time in the order they believe is correct.

Tip: Ask "why that order?" before revealing the answer on a wrong attempt — repeat until 3 flawless runs in a row.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A technician writes "checked electrical, seemed fine" as their only note on a work order.

A. Vague notes like this are nearly useless if the problem returns or needs review.

### Week B6 · Day 2 — Customer Communication Basics

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Match

Opening

"A perfect diagnosis explained badly still feels like a bad experience to the customer — this is a real skill, not a soft extra."

Theory Script

- 1. Translate technical findings into plain language — "12.1V, below the healthy 12.6V range" becomes "your battery isn't holding charge as well as it should."

- 2. Always explain WHY a repair is needed, not just WHAT it costs — customers trust explanations more than instructions.

- 3. If unsure of something, say so honestly rather than guessing confidently — credibility matters more than sounding certain.

Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

Tip: No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A customer asks a technician a question the technician isn't fully sure about.

A. Honest uncertainty, followed by checking, builds more long-term trust than a confident guess.

### Week B6 · Day 3 — Safety Culture & PPE Fundamentals

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Match

Opening

"General shop safety culture, before Week 1 of the main program narrows this specifically to high-voltage hazards."

Theory Script

- 1. PPE (Personal Protective Equipment) is chosen based on the specific task's hazards — one set of PPE doesn't fit every job.

- 2. Safety culture means speaking up when you see a risk, even if it's not your task — this was practiced back in Week B1 Day 2.

- 3. A near-miss (something almost went wrong) is worth reporting and discussing, not hiding — it's free information about a real risk.

- PPE = Personal Protective Equipment — gear selected for the specific hazards of a task (e.g. safety glasses, gloves, closed-toe shoes).

Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

Tip: No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. You notice a near-miss — a tool almost fell on a classmate's foot, but didn't.

A. A near-miss is valuable information about a real risk — reporting it prevents a future actual injury.

### Week B6 · Day 4 — Comprehensive Practice Scenario

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Branch

Opening

"One last practice run combining everything from all 4 bridge weeks before tomorrow's readiness check."

Theory Script

- 1. This scenario deliberately combines systems thinking, safety, documentation, Ohm's Law, diagnostic reasoning, and the sensor-actuator-ECU relationship.

- 2. If any part of this feels unfamiliar, that's useful information for tomorrow's readiness check — better to know now than in Week 1 of the main program.

Simulator Facilitation

How it works: A situation is presented; choosing an option leads to the next situation and feedback.

Tip: Get them fully immersed as "the technician." Ask what made them choose that option before reading the feedback.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. Looking at this scenario, why did the burning smell change the order of steps?

A. Safety-relevant symptoms like an unusual smell take priority over the normal step order.

### Week B6 · Day 5 — Readiness Assessment — Gate to Week 1

60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  Simulator: Sort

Opening

"This is an honest checkpoint, not a formality — the goal is to confirm you're genuinely ready for the pace and assumptions of Week 1 of the main EV program."

Theory Script

- 1. The main 12-week EV program assumes, without re-explaining, everything covered in these 4 bridge weeks: systems thinking, shop safety, documentation, Ohm's Law, the 4-step diagnostic pattern, multimeter use, DTC basics, and the sensor-actuator-ECU relationship.

- 2. If you're not yet confident in one or more of these areas, the right move is to repeat that specific bridge week, not to push ahead and struggle silently.

- 3. This is exactly the same principle used throughout the OJT phase of the main program: a weak item takes priority over pace.

Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

Tip: State in one sentence why that's the correct category right after a wrong answer.

Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

Common Question

Q. A student is unsure about Ohm's Law but says nothing and wants to move on to Week 1 anyway.

A. Flagging a weak area now, before Week 1 builds on it, is exactly the right move — the same principle as the main program's OJT phase.

# After This Track: Transition to the Main EV Program

Graduates who pass the Week B6 Day 5 readiness gate should move directly into Week 1 of the 12-week EV Diagnosis & Repair program. No further onboarding is needed — Week 1 of the main program can proceed at its normal pace for these graduates.

Consider having bridge-track graduates who performed strongly assist as informal peer mentors for the next bridge cohort — the same standardization principle used in the main program (learners becoming teachers).