SMK Bridge Track — ICE Fundamentals Pre-Requisite

Instructor Manual (4 Weeks / 20 Sessions) — Companion to the 12-Week EV Diagnosis & Repair Program

# Why This Track Exists

This is not a shortened version of the main 12-week EV program. It is a separate, purpose-built pre-requisite for cohorts who don't yet have ICE-diagnostic-technician-level skills — most notably SMK (Indonesian vocational high school) students, who typically have ICE fundamentals from coursework but are not yet working diagnostic technicians.

Do not use this track for cohorts that already meet the ICE-technician prerequisite (e.g. Toyota-style novice technicians already employed post-training) — for that audience, start directly at Week 1 of the main EV program; this bridge track would be redundant and could feel condescending.

💡 Rule of thumb: if a cohort has already worked as a paid or apprenticed automotive technician, skip this bridge track. If they are current students without professional technician experience, run this track first.

# Structure at a Glance

| Week | Focus | Why It Matters Later |
| --- | --- | --- |
| B1 | Vehicle systems, shop safety, documentation, Ohm's Law | Every later week assumes this vocabulary and math without re-explaining it |
| B2 | 4-step diagnostic pattern, multimeter use, DTC basics | This IS the diagnostic thinking pattern used throughout the entire 12-week EV program |
| B3 | 12V battery/charging, wiring diagrams, sensor-actuator-ECU | The sensor-ECU-actuator loop is exactly the EV program's BMS concept, just at low voltage first |
| B4 | Documentation, communication, safety culture, readiness gate | Confirms genuine readiness before Week 1 of the main program, not just completion |

# The Week B4 Day 5 Readiness Gate — Read This Carefully

This is the most important instructional decision point in the bridge track. Day 5 is not a formality — it is an honest checkpoint.

💡 If a participant is not genuinely confident in one or more of the 4 bridge weeks, the correct move is to have them repeat that specific week, not to push them into Week 1 of the main EV program. This is the exact same "a weak item takes priority over pace" principle used throughout the main program's OJT phase (Weeks 9-12) — apply it here too.

A participant who begins Week 1 of the main EV program without a solid foundation in all 4 bridge weeks will experience compounding confusion, because Week 1 onward assumes this material without re-explaining it. Letting someone through the gate as a courtesy does them a disservice.

# Session-by-Session Scripts (20 Sessions)

## Week 1 (B1) — Vehicle Systems & Shop Fundamentals

🎯 Goal: build the baseline every later week assumes — how a vehicle's systems fit together, general shop safety, reading documentation, and basic electrical math.

### Week 1 · Day 1 — How a Vehicle Works — the Big Systems Picture

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🗂 Sort

🎬 Opening

"Before diagnosing anything, you need a mental map of what's actually in a car and how the pieces talk to each other."

📘 Theory Script

- A vehicle is made of major systems that interact: engine/powertrain, electrical, chassis/suspension, braking, and body/interior.

- Diagnostic thinking always starts by asking "which system is this symptom coming from?" before going any deeper.

- Most modern vehicles also have a central computer (ECU) that many systems report to — this becomes very important later.

- A technician's job is rarely to fix one bolt — it's to correctly identify which system owns the problem first.

- ECU = Electronic Control Unit — the vehicle's onboard computer that monitors and controls many systems.

💡 Analogy: think of a car like a hospital. The ECU is the reception desk that routes a patient (a symptom) to the right department (system) — engine, electrical, brakes, etc.

🎮 Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

💡 State in one sentence why that's the correct category right after a wrong answer.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A customer says the car "just feels weak" when accelerating, no warning lights.

A. Loss of power under acceleration points to the engine/powertrain system first.

### Week 1 · Day 2 — Hand Tools & General Shop Safety

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🎯 Select

🎬 Opening

"Before touching any vehicle, a shop has non-negotiable safety habits — this is the foundation Week 1 of the EV program builds on."

📘 Theory Script

- Always wear safety glasses and closed-toe shoes in the shop — no exceptions, regardless of the task.

- Use the correct tool for the job — using the wrong size wrench is a leading cause of stripped bolts and hand injuries.

- Keep your workspace organized — tools left on the floor or engine bay are a tripping/dropping hazard.

- Never work under a vehicle that is not properly supported on rated jack stands — a jack alone is not a support device.

- Jack stand = A mechanical support device rated to hold a vehicle's weight safely — different from a jack, which only lifts.

🎮 Simulator Facilitation

How it works: Multiple items are shown; the participant selects the correct one(s) and submits.

💡 Tell them not to submit until confident — this type sees a lot of rushed, wrong submissions.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A classmate is about to slide under a car that's held up by a jack only, no stands.

A. A jack alone can fail — jack stands are mandatory before anyone goes underneath.

### Week 1 · Day 3 — Reading a Service Manual & Technical Documentation

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔗 Match

🎬 Opening

"Every later week in this program assumes you can find and read the right page in a manual — this is that skill, explicitly."

📘 Theory Script

- A service manual is organized by system (engine, electrical, brakes, etc.) — knowing the system name from Day 1 is what lets you find the right chapter fast.

- Manuals use standardized symbols and abbreviations — learning to recognize a handful of common ones saves huge amounts of time.

- Specification tables (torque values, resistance ranges, fluid capacities) are the most-referenced part of any manual — always confirm against them rather than guessing.

- When in doubt about a spec, the manual is always right over memory or assumption.

- Specification (spec) = An exact value (e.g. torque, resistance, capacity) that a manufacturer requires for a correct repair.

💡 Analogy: a service manual is like a recipe book — skipping the exact measurements (specs) is how recipes (repairs) go wrong.

🎮 Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

💡 No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. You're not sure of the exact torque value for a bolt, but you remember roughly what a similar bolt needed.

A. Always confirm exact specs in the manual rather than relying on memory.

### Week 1 · Day 4 — Basic Electrical Fundamentals — Voltage, Current, Resistance

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 📟 Meter Read

🎬 Opening

"This is the single most important day in the bridge track — every EV week later assumes you're comfortable with this math."

📘 Theory Script

- Voltage (V) is electrical pressure — what pushes current through a circuit. Measured in volts.

- Current (A) is the flow of electricity — measured in amps. Current only flows when there's a complete circuit.

- Resistance (Ω) opposes current flow — measured in ohms. More resistance means less current for the same voltage.

- Ohm's Law ties them together: Voltage = Current × Resistance (V = I × R). This one formula underlies almost every electrical diagnostic calculation you'll do later.

- Ohm's Law = V = I × R. Voltage equals current multiplied by resistance — the foundational formula of electrical diagnosis.

💡 Analogy: voltage is like water pressure, current is like water flow rate, and resistance is like how narrow the pipe is. A narrower pipe (more resistance) means less flow (current) for the same pressure (voltage).

🎮 Simulator Facilitation

How it works: A gauge reading is shown; the participant judges normal/abnormal.

💡 Getting the right answer matters less than making them explain WHY — have them recompute the value themselves.

📝 Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A circuit shows 24V and you measure 2A of current flowing.

A. R = V/I = 24V / 2A = 12Ω.

### Week 1 · Day 5 — Week B1 Wrap-Up — Systems & Safety Review

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🗂 Sort

🎬 Opening

"Before moving to diagnostic reasoning, let's confirm this week's foundation is solid."

📘 Theory Script

- This week covered: vehicle systems overview, shop safety, reading documentation, and Ohm's Law.

- Every one of these four topics gets used, without re-explanation, starting next week — this is the last checkpoint before the pace picks up.

🎮 Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

💡 State in one sentence why that's the correct category right after a wrong answer.

📝 Written Test

3 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A new student asks you why Ohm's Law matters for a car that isn't even electric.

A. Every vehicle has electrical systems (starter, lights, sensors) that follow Ohm's Law, not just EVs.

## Week 2 (B2) — Basic Diagnostic Reasoning

🎯 Goal: build the symptom→cause→verify thinking pattern that every later diagnostic week in the main program assumes you already have.

### Week 2 · Day 1 — What Is Diagnosis? Symptom → Cause → Verify

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔢 Sequence

🎬 Opening

"Diagnosis isn't guessing — it's a repeatable thinking pattern. Today you learn the pattern itself, before any specific system."

📘 Theory Script

- Step 1 — Symptom: what does the customer or data actually report? Get this precise before anything else.

- Step 2 — Possible causes: list plausible causes for that exact symptom, from most to least likely.

- Step 3 — Test: check the most likely, lowest-cost cause first — don't jump to the expensive part.

- Step 4 — Verify: after a fix, confirm the original symptom is actually gone, not just assumed fixed.

- Low-cost check = A cheap, fast test performed before expensive or time-consuming ones — check simple things first.

💡 Analogy: a doctor doesn't order surgery before checking your temperature and asking what hurts. Diagnosis always starts cheap and simple, then narrows down.

🎮 Simulator Facilitation

How it works: Show the items in random order; the participant clicks them one at a time in the order they believe is correct.

💡 Ask "why that order?" before revealing the answer on a wrong attempt — repeat until 3 flawless runs in a row.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A customer says "the car makes a weird noise sometimes."

A. A vague symptom must be made precise before you can list plausible causes.

### Week 2 · Day 2 — Using a Multimeter Safely (12V Practice)

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔢 Sequence

🎬 Opening

"The multimeter is the single most-used diagnostic tool — and this 12V practice is what makes the EV program's later HV meter work feel familiar instead of brand new."

📘 Theory Script

- A multimeter can measure voltage, current, and resistance — always set it to the correct mode before connecting it.

- For voltage measurement, connect the meter in parallel (across the two points) — never in series.

- Always start on the highest range setting if unsure of the expected value, then narrow down — this protects the meter from damage.

- Confirm the meter itself is working (test on a known source) before trusting a 'no reading' result — it might be a broken meter, not a broken circuit.

- Parallel connection = Connecting a meter across two points of a circuit to measure voltage, without breaking the circuit.

🎮 Simulator Facilitation

How it works: Show the items in random order; the participant clicks them one at a time in the order they believe is correct.

💡 Ask "why that order?" before revealing the answer on a wrong attempt — repeat until 3 flawless runs in a row.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. You connect a multimeter set to voltage mode and get no reading (0V) on a circuit you expected to have 12V.

A. Always confirm the meter is working before trusting an unexpected reading.

### Week 2 · Day 3 — Reading OBD-II / DTC Codes (ICE-Level Introduction)

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔗 Match

🎬 Opening

"Diagnostic trouble codes exist on every modern vehicle, not just EVs — this is the gentle on-ramp before the EV program's own DTC system."

📘 Theory Script

- A DTC (Diagnostic Trouble Code) is a code the ECU stores when it detects something outside normal range.

- Codes are typically structured by letter + numbers — the letter tells you which system (e.g. P for Powertrain).

- A code tells you where the ECU noticed a problem — it does NOT automatically tell you the root cause. Further diagnosis is still required.

- Clearing a code without fixing the underlying cause means it will very likely come back.

- DTC = Diagnostic Trouble Code — a stored code indicating the ECU detected a reading outside normal range in a specific system.

💡 Analogy: a DTC is like a smoke alarm going off — it tells you where the smoke is, not necessarily what's burning. You still have to go look.

🎮 Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

💡 No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A technician reads a DTC, then immediately clears it without further checks and tells the customer it's fixed.

A. A code is a starting point for diagnosis, not a fix — the underlying cause must still be found and corrected.

### Week 2 · Day 4 — Basic Mechanical Fault-Finding Practice

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🌳 Branch

🎬 Opening

"Combining the 4-step diagnostic pattern with a real (simple) mechanical scenario."

📘 Theory Script

- Mechanical symptoms often show a pattern: constant vs. intermittent, and load-dependent vs. not — noting this narrows causes fast.

- Visual inspection (leaks, loose parts, wear marks) is almost always a valid low-cost first check before any tool comes out.

- Comparing to a known-good reference (another vehicle, a spec sheet) helps you recognize what 'abnormal' actually looks like.

🎮 Simulator Facilitation

How it works: A situation is presented; choosing an option leads to the next situation and feedback.

💡 Get them fully immersed as "the technician." Ask what made them choose that option before reading the feedback.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A noise happens constantly, regardless of speed, load, or road surface.

A. A constant, condition-independent noise rules out bump- or load-triggered causes, redirecting your search.

### Week 2 · Day 5 — Week B2 Wrap-Up — Diagnostic Reasoning Review

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🗂 Sort

🎬 Opening

"This week's thinking pattern gets reused, unexplained, in every diagnostic week from here forward — let's confirm it's solid."

📘 Theory Script

- This week covered: the 4-step diagnostic pattern, safe multimeter use, DTC code basics, and applying it all to a mechanical scenario.

- Notice that none of this was EV-specific — that's intentional. This is the general skill the EV-specific weeks build on top of.

🎮 Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

💡 State in one sentence why that's the correct category right after a wrong answer.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A classmate says DTC codes make diagnosis unnecessary since "the code tells you what's wrong."

A. This is a common misconception worth correcting clearly.

## Week 3 (B3) — Electrical & Electronic Systems

🎯 Goal: build comfort with 12V battery/charging systems, wiring diagrams, and the sensor-actuator-ECU relationship — the direct conceptual bridge into the EV program's BMS-heavy content.

### Week 3 · Day 1 — 12V Battery & Charging System Basics

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 📟 Meter Read

🎬 Opening

"Every ICE vehicle already has a small-scale version of the battery/charging concepts EV Week 2 will build on."

📘 Theory Script

- The 12V battery stores energy and provides high current briefly for starting the engine.

- The alternator recharges the battery and powers electrical systems once the engine is running.

- A battery's state can be judged by voltage: roughly 12.6V+ = healthy resting charge, below ~12.0V = weak/discharged.

- A charging system fault (bad alternator) often shows up as a battery that drains repeatedly, not just once.

- Alternator = A generator driven by the engine that recharges the battery and powers electrical loads while the engine runs.

🎮 Simulator Facilitation

How it works: A gauge reading is shown; the participant judges normal/abnormal.

💡 Getting the right answer matters less than making them explain WHY — have them recompute the value themselves.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A customer's battery keeps going dead every few days, even after being recharged each time.

A. Repeated draining despite recharging points to the charging system, not just the battery itself.

### Week 3 · Day 2 — Reading Wiring Diagrams

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔢 Sequence

🎬 Opening

"Wiring diagrams look intimidating at first — today breaks them into the handful of symbols that cover most of what you'll see."

📘 Theory Script

- A wiring diagram shows components as symbols connected by lines representing wires, often color-coded.

- Following a diagram means tracing a path from power source, through a component, to ground — a circuit needs all three.

- Wire colors and gauge (thickness) are usually labeled and matter — swapping a thin wire for a thick one's job can cause overheating.

- Ground symbols matter as much as power symbols — a broken ground is a very common, easy-to-miss fault.

- Ground = The return path of a circuit back to the vehicle's chassis/battery negative — required to complete any circuit.

💡 Analogy: a circuit is like a water loop — power is the pump pushing water out, the component is a wheel the water turns, and ground is the pipe returning water back to the pump. Break any part and the loop stops.

🎮 Simulator Facilitation

How it works: Show the items in random order; the participant clicks them one at a time in the order they believe is correct.

💡 Ask "why that order?" before revealing the answer on a wrong attempt — repeat until 3 flawless runs in a row.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A component isn't working, and you've confirmed power is reaching it correctly.

A. A confirmed-good power supply doesn't rule out a broken ground — both are required for the circuit.

### Week 3 · Day 3 — Sensors & Actuators — How They Talk to the ECU

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🗂 Sort

🎬 Opening

"This is the conceptual bridge day — the EV program's BMS and sensor-heavy content assumes you already understand this relationship."

📘 Theory Script

- A sensor measures something (temperature, speed, position) and reports it to the ECU as an electrical signal.

- An actuator receives a command from the ECU and does something physical (opens a valve, turns a motor, lights an indicator).

- The ECU's job is to read sensors, make a decision based on programmed logic, and command actuators in response.

- If a sensor sends a wrong or missing signal, the ECU can make a wrong decision — this is why sensor faults often cause seemingly unrelated symptoms.

- Sensor = A component that measures a physical condition and converts it into an electrical signal for the ECU.

- Actuator = A component that receives an ECU command and performs a physical action.

💡 Analogy: sensors are the ECU's eyes and ears, actuators are its hands. The ECU itself is the brain deciding what to do with what it senses.

🎮 Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

💡 State in one sentence why that's the correct category right after a wrong answer.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A cooling fan doesn't turn on even though the engine is clearly overheating.

A. Both a sensor fault (wrong input) and an actuator fault (bad output) can cause the same visible symptom — this is why diagnosis matters.

### Week 3 · Day 4 — Combined Electrical + Mechanical Troubleshooting Practice

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🗂 Sort

🎬 Opening

"Real-world faults don't announce whether they're electrical or mechanical — today combines everything from Weeks B1-B3."

📘 Theory Script

- Always separate a symptom into possible electrical vs. mechanical causes before testing — this keeps your search organized.

- A quick way to start separating: does the fault change with vibration/movement (often mechanical/connection) or is it constant (often a steady electrical fault)?

- Use the low-cost checks first, then the multimeter, then deeper inspection — the same order regardless of system.

🎮 Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

💡 State in one sentence why that's the correct category right after a wrong answer.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A gauge that normally moves is now stuck at exactly zero, all the time, regardless of conditions.

A. A hard stop at exactly zero often suggests a broken connection or total signal loss, not a gradual sensor drift.

### Week 3 · Day 5 — Week B3 Wrap-Up — Electrical Systems Review

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔗 Match

🎬 Opening

"This week's sensor-actuator-ECU concept in particular will resurface constantly in the EV program's BMS content."

📘 Theory Script

- This week covered: 12V battery/charging basics, wiring diagram reading, the sensor-actuator-ECU relationship, and combined troubleshooting.

- The sensor → ECU → actuator loop from Day 3 is the single most important concept to carry forward — the EV program's BMS is exactly this same loop, just at high voltage.

🎮 Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

💡 No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. Looking back at this week, which single concept do you think will matter most for understanding an EV's BMS later?

A. The BMS is fundamentally the same sensor-decision-actuator loop you learned this week, just applied to high-voltage battery cells.

## Week 4 (B4) — Professional Readiness & Gate Check

🎯 Goal: build professional habits (documentation, communication, safety culture) and confirm — with an honest gate check — readiness to begin Week 1 of the main 12-week EV program.

### Week 4 · Day 1 — Documentation & Work Orders

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔢 Sequence

🎬 Opening

"Diagnosis that isn't written down might as well not have happened — professional habits start with recordkeeping."

📘 Theory Script

- A work order should record: the reported symptom, tests performed, findings, and the fix applied — in that order.

- Write findings precisely, not vaguely — "12.1V, below normal" is useful; "battery seemed weak" is not.

- Good documentation protects both the technician and the customer if a problem returns or is disputed later.

- Work order = The formal record of a repair job: reported symptom, diagnostic steps, findings, and repair performed.

🎮 Simulator Facilitation

How it works: Show the items in random order; the participant clicks them one at a time in the order they believe is correct.

💡 Ask "why that order?" before revealing the answer on a wrong attempt — repeat until 3 flawless runs in a row.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A technician writes "checked electrical, seemed fine" as their only note on a work order.

A. Vague notes like this are nearly useless if the problem returns or needs review.

### Week 4 · Day 2 — Customer Communication Basics

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔗 Match

🎬 Opening

"A perfect diagnosis explained badly still feels like a bad experience to the customer — this is a real skill, not a soft extra."

📘 Theory Script

- Translate technical findings into plain language — "12.1V, below the healthy 12.6V range" becomes "your battery isn't holding charge as well as it should."

- Always explain WHY a repair is needed, not just WHAT it costs — customers trust explanations more than instructions.

- If unsure of something, say so honestly rather than guessing confidently — credibility matters more than sounding certain.

🎮 Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

💡 No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A customer asks a technician a question the technician isn't fully sure about.

A. Honest uncertainty, followed by checking, builds more long-term trust than a confident guess.

### Week 4 · Day 3 — Safety Culture & PPE Fundamentals

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🔗 Match

🎬 Opening

"General shop safety culture, before Week 1 of the main program narrows this specifically to high-voltage hazards."

📘 Theory Script

- PPE (Personal Protective Equipment) is chosen based on the specific task's hazards — one set of PPE doesn't fit every job.

- Safety culture means speaking up when you see a risk, even if it's not your task — this was practiced back in Week B1 Day 2.

- A near-miss (something almost went wrong) is worth reporting and discussing, not hiding — it's free information about a real risk.

- PPE = Personal Protective Equipment — gear selected for the specific hazards of a task (e.g. safety glasses, gloves, closed-toe shoes).

🎮 Simulator Facilitation

How it works: Click a concept on the left and its matching description on the right.

💡 No penalty for wrong guesses — let them experiment, then explain WHY they paired things that way.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. You notice a near-miss — a tool almost fell on a classmate's foot, but didn't.

A. A near-miss is valuable information about a real risk — reporting it prevents a future actual injury.

### Week 4 · Day 4 — Comprehensive Practice Scenario

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🌳 Branch

🎬 Opening

"One last practice run combining everything from all 4 bridge weeks before tomorrow's readiness check."

📘 Theory Script

- This scenario deliberately combines systems thinking, safety, documentation, Ohm's Law, diagnostic reasoning, and the sensor-actuator-ECU relationship.

- If any part of this feels unfamiliar, that's useful information for tomorrow's readiness check — better to know now than in Week 1 of the main program.

🎮 Simulator Facilitation

How it works: A situation is presented; choosing an option leads to the next situation and feedback.

💡 Get them fully immersed as "the technician." Ask what made them choose that option before reading the feedback.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. Looking at this scenario, why did the burning smell change the order of steps?

A. Safety-relevant symptoms like an unusual smell take priority over the normal step order.

### Week 4 · Day 5 — Readiness Assessment — Gate to Week 1

⏱ 60 min (Review 5 · Theory 15 · Simulator 20 · Written Test 10 · Discussion 10)  |  🎯 Simulator: 🗂 Sort

🎬 Opening

"This is an honest checkpoint, not a formality — the goal is to confirm you're genuinely ready for the pace and assumptions of Week 1 of the main EV program."

📘 Theory Script

- The main 12-week EV program assumes, without re-explaining, everything covered in these 4 bridge weeks: systems thinking, shop safety, documentation, Ohm's Law, the 4-step diagnostic pattern, multimeter use, DTC basics, and the sensor-actuator-ECU relationship.

- If you're not yet confident in one or more of these areas, the right move is to repeat that specific bridge week, not to push ahead and struggle silently.

- This is exactly the same principle used throughout the OJT phase of the main program: a weak item takes priority over pace.

🎮 Simulator Facilitation

How it works: One item at a time; the participant clicks the correct bucket.

💡 State in one sentence why that's the correct category right after a wrong answer.

📝 Written Test

2 multiple-choice questions. Re-explain any wrong answer immediately using this day's theory points.

❓ Common Question

Q. A student is unsure about Ohm's Law but says nothing and wants to move on to Week 1 anyway.

A. Flagging a weak area now, before Week 1 builds on it, is exactly the right move — the same principle as the main program's OJT phase.

# After This Track: Transition to the Main EV Program

Graduates who pass the Week B4 Day 5 readiness gate should move directly into Week 1 of the 12-week EV Diagnosis & Repair program. No further onboarding is needed — Week 1 of the main program can proceed at its normal pace for these graduates.

Consider having bridge-track graduates who performed strongly assist as informal peer mentors for the next bridge cohort — the same standardization principle used in the main program (learners becoming teachers).